# Preparing a risk management and monitoring program

# Appendix C – Example of a monitoring plan

| Environmental element | Monitoring activity | Frequency | Critical control/s | Environmental indicators | Environmental performance objectives | Link to monitoring record |
| --- | --- | --- | --- | --- | --- | --- |
| Groundwater | Groundwater monitoring of all site bores (MW01-MW10) | Biannually | Bunding | Water quality indicators: pH, DO, EC, redox CoPCs: PFAS, TPH, VOCs, heavy metals, nitrate | Prevent contaminants potential of concern from polluting groundwater | Groundwater monitoring trending analysis spreadsheet.xlsx Trigger action response on groundwater quality |
| Groundwater | Inspections | Weekly | Bunding | Integrity and condition of the bunding, such as: - no visible cracks - seal intact - no accumulated liquid - bund release valve shut | Prevent contaminants potential of concern from polluting groundwater | Inspection records |
| Odour | Continuous operating parameters monitoring | Real time monitoring | Biofilter | Temperature, humidity, pH, moisture, back pressure | To maintain optimal conditions in the biofilter to support microbial activity | SCADA data |
| Odour | Biofilter performance assessment | Quarterly | Biofilter | Flow distribution across surface of biofilter | Less than +/- 25% variation in flow across sections of biofilter surface | Biofilter flow testing spreadsheet |
| Odour | Biofilter performance assessment | Annually | Biofilter | Odour concentration and character | <500 Odour units concentration, neutral or earthy odour character | Stack testing report |
| Air quality | Continuous emissions monitoring of DP01, DP02 and DP03 | Real time monitoring | Water injection (to minimise NOX) | NOX | Reduce the emissions of NOx by 30% within the next 12 months compared to 2022/23 air monitoring results | SCADA data |
| Air quality | Stack testing | Annually | Wet scrubber | SO2 | Reduce the emissions of SO2 by 30% within the next 12 months compared to 2022/23 air monitoring results | Stack testing reports |
| Surface water | Wastewater monitoring program | Real time monitoring | Dissolved air flotation | Water quality parameters: pH, turbidity, conductivity, DO | Achieve a minimum 90% reduction in Chemical Oxygen Demand (COD), Total Suspended Solids (TSS), and Total Nitrogen (TN) in treated wastewater, and ensure final effluent concentrations remain below 50 mg/L COD, 20 mg/L TSS, and 10 mg/L TN prior to offsite discharge. Reduce organic matter, TSS and nutrient load in treated wastewater by 90% before it is discharged offsite | Laboratory results Wastewater analysis spreadsheet |
| Surface water | Wastewater monitoring program | Weekly sampling | Dissolved air flotation | COD, BOD, ammonia, nutrients | Achieve a minimum 90% reduction in Chemical Oxygen Demand (COD), Total Suspended Solids (TSS), and Total Nitrogen (TN) in treated wastewater, and ensure final effluent concentrations remain below 50 mg/L COD, 20 mg/L TSS, and 10 mg/L TN prior to offsite discharge. Reduce organic matter, TSS and nutrient load in treated wastewater by 90% before it is discharged offsite | Laboratory results Wastewater analysis spreadsheet |

Notes

Your monitoring activity may be hyperlinked to indicate where the monitoring program may be found

The monitoring activity is linked back to the critical control

Monitoring activities are linked to relevant environmental performance objectives