Publication 5008

April 2026

|  | Authorised and published by the Victorian Government, 1 Treasury Place, Melbourne |
| --- | --- |

# Calculation of financial assurance for prescribed permission activities

epa.vic.gov.au

Environment Protection Authority Victoria

GPO BOX 4395 Melbourne VIC 3001

1300 372 84

This content is for general information only. Obtain professional advice if you have any specific concern. EPA Victoria has made reasonable effort to ensure accuracy at the time of publication. Except where noted at epa.vic.gov.au/copyright, all content in this work* is licensed under the Creative Commons Attribution 4.0 Licence. To view a copy of this licence, visit creativecommons.org. EPA acknowledges Victoria’s First Nations peoples as the Traditional Owners of the land and water on which we live and work. We pay our respect to their Elders past and present.

Contents

Purpose of this guideline	5

1	Introduction	6

1.1	Prescribed permission activities	6

1.2	Risk assessment criteria	6

2	Determining the amount of financial assurance	7

3	Calculating financial assurance for landfills	8

3.1	Assessment by an environmental auditor	8

3.2	Documentation for the calculation of financial assurance	8

3.3	Calculating the operational component of landfill financial assurance	9

3.3.1	Release of the operational component of financial assurance	9

3.4	Calculating the closure and aftercare components of landfill financial assurance	10

3.4.1	Calculation of capping costs	11

3.5	Progressive release of closure financial assurance	11

3.6	Calculating aftercare financial assurance	11

3.7	Progressive release of aftercare financial assurance	12

3.8	The final release of financial assurance	12

4	Calculating financial assurance for reportable priority waste management	13

4.1	Reportable priority waste disposal costs	13

4.1.1	About disposal costs	13

4.1.2	Containers contaminated with reportable priority waste	14

4.1.3	Contaminated soil	15

4.2	Organic waste processing of reportable priority waste	15

5	Calculating financial assurance for waste and resource recovery facilities	16

5.1	Calculation method	16

5.1.1	Assumptions and considerations for calculating financial assurance for WRRFs	16

6	Calculating financial assurance for bulk storage	18

6.1	Activities and assumptions for calculating financial assurance for bulk storage	18

6.2	Site specific considerations and minimum cost information	18

7	Calculating financial assurance for contaminated sites – on-site soil retention	25

7.1	Assessment by an environmental auditor	25

7.2	Activities for calculating financial assurance for contaminated sites - on-site soil retention	25

7.3	Site specific considerations and minimum cost information	25

Appendix A – Calculating a Consumer Price Index adjustment for financial assurance – reportable priority waste management	28

Appendix B – Calculating a Consumer Price Index adjustment for financial assurance – waste resource recovery facilities	29

# Purpose of this guideline

This guide sets out Environment Protection Authority (EPA) Victoria’s method for calculating the amount of financial assurance that may be required as a condition of a development licence, operating licence or permit under the Environment Protection Act 2017 (the Act), for:

- landfills

- reportable priority waste management (RPW)

- waste and resource recovery facilities (WRRF)

- bulk storage

- contaminated sites - on-site soil retention

Read more about financial assurance:

- Financial assurance for permissions and contaminated land management (publication 2002) (https://www.epa.vic.gov.au/20021-financial-assurance-permissions-and-contaminated-land-management) explains the laws and requirements for financial assurance.

- Forms of financial assurance (publication 1595) (https://www.epa.vic.gov.au/15952-forms-financial-assurance) provides an overview of the different forms of financial assurance that EPA may consider and in what circumstances.

- EPA’s website provides information about financial assurances and the risk assessment criteria - https://www.epa.vic.gov.au/financial-assurance

# Introduction

Financial assurance was introduced to Victoria’s environment protection legislation following several incidents where we conducted cleanup following business failures, waste abandonment and pollution events. Financial assurance ensures that in the event remediation or cleanup is required, funds are available.

We may require financial assurance as a condition of

- a prescribed permission

- an environmental action notice

- site management order

- an Order relating to environmentally hazardous substances.

This is listed in section 219(1) of the Act.

## Prescribed permission activities

Only certain activities from Schedule 1 of the Environment Protection Regulations 2021 (the Regulations) may be required to provide financial assurance as a condition of a permission. These are set out in regulation 167. They are:

  - Reportable priority waste management (A01)

  - Landfills - excluding municipal landfills servicing <5000 people (A05a)

  - Waste and resource recovery – large (A13a)

  - Waste and resource recovery - medium (A13b)

  - Bulk storage (G04)

  - Contaminated sites – on-site soil retention (L02)

## Risk assessment criteria

We will apply the risk assessment criteria set out in regulation 168 to determine:

- if a financial assurance is required for a prescribed permission

- the appropriate form of the financial assurance.

Read more about the risk assessment criteria on EPA’s website - https://www.epa.vic.gov.au/financial-assurance.

# Determining the amount of financial assurance

We will have regard to the factors set out in section 221 of the Act when determining the amount of financial assurance. These are:

  - a reasonable estimate of costs and expenses of remediation or cleanup activities

  - any method for calculating the amount of financial assurance that we publish

  - an independent assessment of the amount of financial assurance, if we require it under section 222 of the Act.

This guide constitutes methods published by EPA for calculating the amount of financial assurance.

We will determine the amount of financial assurance for the permission activities prescribed in regulation 167 in accordance with this guide unless:

  - We determine that a tailored financial assurance calculation is required to better estimate the costs and expenses. This could include remediation or clean up associated with the activity or type of waste managed at the site

  - the duty holder can sufficiently demonstrate to EPA us that a different calculation method is appropriate. A different method would only be applicable in exceptional circumstances.

# Calculating financial assurance for landfills

Calculation of financial assurance for landfills has three components:

  - operational

  - closure

  - aftercare.

## Assessment by an environmental auditor

We require an independent assessment of the amount of financial assurance for landfills to be provided by an environmental auditor, appointed under the Act.

Find more information about auditor assessment of financial assurance proposals, and the assessment form at EPA’s website https://www.epa.vic.gov.au/financial-assurance-permissions-and-contaminated-land.

## Documentation for the calculation of financial assurance

Table 1 lists the documents required to support the calculation for financial assurance 
for landfills.

Table 1	Documents for landfill financial assurance

Document description

A map of the landfill showing all the cells at the site. It must identify: - current cell(s) - cells with intermediate capping - partially rehabilitated cells - fully rehabilitated cells, including the dates of rehabilitation and approved cell volumes for each cell.

The audit report identification numbers for audit reports containing as-constructed details of landfill cells (cell designs). These must identify cells constructed to the requirements specified in Siting, design, operation and rehabilitation of landfills (publication 788) (https://www.epa.vic.gov.au/about-epa/publications/788-3).

The audit report identification number for the most recent audit report that verifies: - the area of the landfill that has already been filled - the area that is proposed to be filled.

A list of approved variations to requirements for capping set out in Siting, design, operation and rehabilitation of landfills (publication 788) (https://www.epa.vic.gov.au/about-epa/publications/788-3).

Where a Type 3 cap is proposed, evidence supporting that this type of cap is appropriate. For example evidence of classification of the waste as industrial waste.

The audit report identification numbers for reports containing auditor verification of capping and rehabilitation of cells closed after 2011.

The most recent rehabilitation plan.

Any evidence supporting a variation to the default 30-year aftercare period (used for calculation purposes).

## Calculating the operational component of landfill financial assurance

The operational component of landfill financial assurance funds the costs that may be incurred:

- by us in ensuring the safety of an operational landfill

- where an uninsured event exceeds the operator’s capacity to pay for cleanup or rehabilitation before the landfill enters the post-closure phase.

The events covered by the operational component of financial assurance are difficult to predict but can represent significant costs. It is therefore not possible to accurately describe or calculate the costs of all contingent events. Instead, our formula calculates landfill operational financial assurance based on:

- The formula is based on the approved volume (in cubic metres) of filled cells that are not fully rehabilitated

- plus the volume of approved cells (recorded in the current operating licence) that are not yet filled.

The approved volume of cells represents the scale of the activity at the site and the potential environmental risk.

The formula for operational landfill financial assurance: Operational financial assurance = $0.45/m3 × total cell volume (filled plus approved)+$135,000

The formula uses estimated remediation costs for the following types of events at variably sized landfills. These events were selected as being representative of a wide variety of the following contingent events at landfills that could result in unexpected costs:

  - loss of leachate containment

  - generation of excess leachate

  - illegal dumping

  - slumping of batters

  - overfilling of waste

  - failure or erosion of temporary capping or vegetation

  - gas migration.

This formula:

  - assumes that contingent events do not occur simultaneously.

  - is based on 2015 costs and will be indexed in subsequent years using the Consumer Price Index (CPI) adjustment calculation provided in Appendix A.

### Release of the operational component of financial assurance

The operational component of financial assurance for a cell may no longer required in some circumstances. This may be the case if:

  - The cell is fully rehabilitated with a final revegetated cap. A cell is not considered to be fully rehabilitated if any portion of the cell has only intermediate capping or partially constructed sidewall. Final capping is demonstrated to meet the objectives of Siting, design, operation and rehabilitation of landfills (publication 788) (https://www.epa.vic.gov.au/about-epa/publications/788-3) where phytocapping is used.

  - Landfill gas and leachate management infrastructure and surface water management engineering controls are installed and operating.

  - All licence conditions (and any relevant notice conditions) concerning leachate and gas management are being complied with for that cell.

  - An environmental auditor has verified that the cell complies with all the above requirements, and their report is provided to us. Alternatively, our written approval must be provided for cells that were fully capped before 2011.

## Calculating the closure and aftercare components of landfill financial assurance

The calculation of the closure and aftercare components of landfill financial assurance is based on the costs associated with undertaking activities set out in:

  - Siting, design, operation and rehabilitation of landfills (publication 788) (https://www.epa.vic.gov.au/about-epa/publications/788-3)

  - Closed landfill guidelines (publication 1490) (https://www.epa.vic.gov.au/about-epa/publications/1490-1)

  - Landfill licensing (publication 1323) (https://www.epa.vic.gov.au/about-epa/publications/1323-3).

Table 2 lists the minimum activities that the closure component of financial assurance should be calculated on.

Table 2	Landfill closure activities

Closure activities

Capping the uncapped area and any area with temporary or intermediate capping, as well as any additional works on existing caps as specified in the rehabilitation plan. This includes material costs as well as installation of the materials See more on the calculation of capping costs below

Haulage and purchase of capping material if the latter is not available on-site

Vegetation establishment and management

Implementation of stormwater management control structures

Installation of final gas and leachate management infrastructure

Decommissioning and removal of redundant operational infrastructure

Hydrogeological assessment and/or review and update of the hydrogeological assessment

Landfill gas risk assessment and/or review and update of the landfill gas risk assessment

Environmental monitoring program

Rehabilitation plan for the remaining rehabilitation work required

Development of an aftercare management plan

Auditor assessment of cap and leachate dam

Audit of construction of the cap including final rehabilitation

Auditing of the leachate dam design and construction (if required)

Management supervision, preliminaries and on-costs

### Calculation of capping costs

In areas where clay is not available for capping, the calculation should incorporate the cost of purchasing and hauling clay unless we approve an alternative capping design.

Where we have not approved the final capping design, the design used as a basis for calculating financial assurance will be as follows:

  - If the licence specified a capping standard at the date of cell completion, this standard would apply. Otherwise, the standard at the time of closure applies.

  - For closures after 2001, the standard at the time is a cap design compliance with Siting, design, operation and rehabilitation of landfills (publication 788) (https://www.epa.vic.gov.au/about-epa/publications/788-3). Currently, a cap design prepared in accordance with Landfill licensing (publication 1323) (https://www.epa.vic.gov.au/about-epa/publications/1323-3 )is the standard that applies.

## Progressive release of closure financial assurance

Any landfill area that has been signed off by an environmental auditor as fully capped and rehabilitated (or approved by us in writing if fully capped prior to 2011), does not need to be included in the closure calculation. For us to release the closure component, an environmental auditor must verify that all rehabilitation activities are completed. Final capping includes proven phytocapping is used where relevant to the site.

## Calculating aftercare financial assurance

Table 3 explains the principles for calculating the aftercare component of financial assurance.

Table 3	Principles for calculating aftercare financial assurance

|  | Principle |
| --- | --- |
| Area | Use the total area of the landfill – the filled area plus the currently approved area. If this area cannot be determined from audit reports, use the default area specified in the licence. |
| Time | Use an aftercare period of 30 years from the final closure date of the entire site. A different aftercare period can be used only where sufficient evidence is provided with the calculation proposal. |
| Accounting | Neither discounting nor inflation should be applied. Adjustments for inflation are included in the periodic re-evaluation of financial assurance amounts. If an accumulating, interest-bearing fund is used to provide for closure and aftercare costs, discounting can be incorporated into the determination of pay-in schedules. |

Table 4 lists the minimum activities that the aftercare component of financial assurance should be calculated on.

Table 4	Landfill aftercare activities

Aftercare activities

Operation and maintenance of all structures including capping (and vegetation), wells and bores, and associated pipework

Capping and vegetation maintenance/augmentation for phytocaps

Leachate extraction/collection, treatment, and disposal

Landfill gas extraction and treatment

Environmental monitoring

Infrastructure and leachate pond decommissioning (at the conclusion of the aftercare period)

Inspection, audit and annual reporting costs

Stormwater management and surface water monitoring

## Progressive release of aftercare financial assurance

Progressive release of aftercare financial assurance involves a risk-based calculation that considers the remaining environmental risks at the site.

Table 5 sets out the criteria that must all be met before we may considerer progressively reducing the aftercare financial assurance.

Table 5	Criteria for reducing aftercare financial assurance

| No. | Criteria |
| --- | --- |
| 1 | Evidence is available of progress in actioning all environmental auditor recommendations relating to managing the risk to the environment. |
| 2 | We have not applied any additional remedial notices or sanctions in the previous five years. |
| 3 | The most recent environmental audit report confirms that the risks to the environment associated with aftercare management are being adequately identified, managed and monitored. |
| 4 | The financial assurance in place has covered costs to date, and so the financial assurance for the remaining aftercare activities can be reduced. |

## The final release of financial assurance

The entire closure and aftercare financial assurance is released once we determine that the site no longer poses a risk to human health or the environment.

Read more about the review and release of financial assurance in Financial assurance for permissions and contaminated land management (publication 2002) (https://www.epa.vic.gov.au/20021-financial-assurance-permissions-and-contaminated-land-management).

# Calculating financial assurance for reportable priority waste management

Financial assurance for reportable priority waste (RPW) management is calculated on the cost of disposing of the amount of RPW that is permitted to be stored on the site under an EPA development or operating licence. The financial assurance may be used for cleaning up or remediation where the duty holder fails to undertake their obligations.

Financial assurance for organic waste processing (composting) of RPW is addressed on page 16 of this guide.

## Reportable priority waste disposal costs

The RPW disposal cost is calculated by multiplying the total amount of RPW the facility is permitted to store by the relevant RPW unit disposal cost.

The formula for RPW disposal cost: RPW disposal cost= ∑Amount of RPW × Unit disposal cost where: amount of RPW = the maximum amount of RPW the facility is licensed to store (tonnes) unit disposal cost = the highest unit disposal cost that is associated with the range of RPW permitted to be stored on the site

### About disposal costs

  - Unit disposal costs for specific waste types are provided in Table 6 to Table 9. A default rate of $1,000 per tonne applies for waste types not otherwise specified. For very high-risk waste types, a different disposal cost may be applied.

  - The costs are quoted in 2015 and shall be indexed in subsequent years using the CPI adjustment formula provided in Appendix A.

  - Where the facility is licensed to store several different wastes, the unit disposal cost is based on the RPW with the highest disposal cost.

  - To minimise the amount of financial assurance required, licence holders can apply to vary their licence to specify separate waste limits for high-cost and low-cost wastes. The financial assurance is then calculated by totalling the waste disposal costs for the permitted amount of each waste.

  - A separate calculation of the costs of transport, sampling and site management is not required. The waste disposal costs used in the financial assurance calculation are designed to include these additional costs and are therefore set deliberately higher than commonly charged gate fees.

Table 6	Waste disposal costs for high-cost reportable priority waste

| Waste type (Schedule 5 of the Regulations) | Waste code | Unit disposal cost (2015 dollars) |
| --- | --- | --- |
| Highly odorous organic chemicals (including mercaptans) | M260 | $20,000 per tonne |
| Solvents, oils and materials contaminated with PCB at a concentration of 50 mg per kg or greater | M100 | $10,000 per tonne |
| Solvents, oils and materials contaminated with PCB at a concentration greater than 2 mg per kg and up to 50 mg per kg | M120 | $5,000 per tonne |
| Mercury waste | D120 | $3,000 per tonne |
| Chlorinated or halogenated waste Isocyanate | G150 M220 | $2,000 per tonne |
| Cyanide waste | A100, A110, A130 | $1,500 per tonne |
| Pesticides | H100, H120, H110 | $1,500 per tonne |

Table 7	Waste disposal costs for lower-cost reportable priority waste

| Waste type (Schedule 5 of the Regulations) | Waste code | Unit disposal cost (2015 dollars) |
| --- | --- | --- |
| Oils, hydrocarbons, emulsions Sludges and slurries Residues | J codes T130 H (hazardous) N210, N205, T330 | $500 per tonne |
| Industrial wash waters | L codes categorised as hazardous | $250 per tonne |
| Putrescible/organic wastes | K codes categorised as hazardous | $100 per tonne |

### Containers contaminated with reportable priority waste

For containers contaminated with RPW, the unit disposal costs in Table 8 apply. If the container storage limit in the licence is expressed in weight, the disposal cost relates to the weight of the containers as well as RPW residues.

If the site has customer storage agreements in place that require containers to be returned to their owners, we may consider a lower amount of financial assurance. These customer agreements must be specified in the licence.

Table 8	Waste disposal costs for containers

| Waste type | Waste code | Unit disposal cost (2015 dollars) |
| --- | --- | --- |
| Rigid steel or plastic containers with an original volume less than 200 L contaminated with RPW | N100 | $20 each or $2/kg |
| Rigid steel or plastic containers with an original volume equal to or greater than 200 L contaminated with RPW | N105 | $50 each or $1/kg |

### Contaminated soil

For premises where soil is permitted to be stored and treated and/or where passive remediation of contaminated soil is permitted, a soil treatment cost may be used to calculate financial assurance as specified in Table 9. For very large quantities, a lower rate may apply. A separate storage limit for Category A, B, C, D soil and soil containing asbestos only must be specified in the licence.

We may determine that a lower amount of financial assurance is required if:

  - customer storage agreements are in place for contaminated soil under treatment that specifies the soil is to be returned to the soil producer, and

  - the licence is amended to require the storage agreements to be in place.

Read more about the classification of waste soil online at epa.vic.gov.au.

Table 9	Treatment/disposal costs for priority waste that is soil

| Classification of priority waste that is soil (Schedule 6 of the Regulation) | Waste code (Schedule 5 of Regulations) | Treatment/disposal cost (2015 dollars) |
| --- | --- | --- |
| Category A | N120 | $850 per tonne |
| Category B | N120 | $450 per tonne |
| Category C | N120 | $180 per tonne |
| Category D | N120 | $180 per tonne |
| Soil containing asbestos only | N120 | $200 per tonne |

## Organic waste processing of reportable priority waste

Calculation of financial assurance for organic waste processing (composting) of RPW or RPW-contaminated wastes is based on the cost of offsite disposal.

Two components are potentially relevant to the calculation of financial assurance for RPW organic waste processors are the storage and mixing of RPW:

Reportable priority waste storage

For premises where an RPW storage limit is specified in the licence. This component of financial assurance is calculated in the same manner as for RPW management. This is shown on page 14.

Reportable priority waste mixing

For premises where there is an area or pit for mixing RPW with other materials. The PIW mixing component applies to sites where a mixing area or pit(s) is designated in the licence. This component is calculated by multiplying the volume of the mixing pit(s) by the waste disposal costs for the most expensive waste type that is permitted to be mixed into the organic waste for processing (composting).

The total financial assurance is calculated by adding the financial assurance components that are relevant to the premises.

# Calculating financial assurance for waste and resource recovery facilities

Financial assurance may be required for medium and large waste and resource recovery facilities (WRRFs). The scale of the waste activity and the type of waste handled at a facility determines the activity type and type of permission required.

Table 10 sets out the threshold values for these activity types as prescribed in Schedule 1 of 
the Regulations.

Table 10	Prescribed permission activities for waste resource recovery that may require financial assurance

| Activity type (Schedule 1 of the Regulations) | Description of prescribed permission activity | Prescribed permission type |
| --- | --- | --- |
| A13a Waste and resource recovery -large | 4,000 tonnes or more combustible, recyclable waste received in one month or 10,000m3 of waste stored on the premises at any time | Development licence, operating licence |
| A13b Waste and resource recovery -medium | - 4,000 tonnes or more of waste excluding combustible, recyclable waste received in one month or - 10,000m3 of waste stored on the premises at any time. Less than 4,000 tonnes of waste (including specified combustible, recyclable waste) received in any month and, between 5,000 m3 and 10,000 m3 of waste stored on the premises at any time. | Development licence, permit |

## Calculation method

If we determines that financial assurance is required for a WRRF, the method to calculate the amount of financial assurance is based on the cost of disposing of waste to landfill. This method is representative of the various cleanup scenarios that may arise at a WRRF. It factors in:

- the scale of the waste activity

- the type of waste handled at a facility.

### Assumptions and considerations for calculating financial assurance for WRRFs

  - Gate fees for landfills are $200.00 per tonne of waste, regardless of the location of the waste resource recovery facility (metro or regional).

  - If the maximum amount of waste permitted to be stored at the site is specified in volume in the permission, we will apply a default density conversion factor of 186 kg/m3 to convert the maximum storage volume of waste into weight unless the permission conditions narrowly restrict the types of waste that can be accepted.

  - Comingled waste consisting of combustible and non-combustible waste will be treated as combustible for the purposes of calculating financial assurance.

  - Fees and costs are quoted in 2021 and shall be indexed in subsequent years using the CPI adjustment formula provided in Appendix B.

Where non commingled waste would likely not be disposed of to landfill but rather transported to another location for recycling or recovery, the formula below may apply with the removal of the landfill gate fees. We may, on an activity and site-specific basis, consider cost additions in the calculation of the financial assurance amount to cover receival charges that may apply at the alternative recycling or recovery.

The formula for financial assurance for WRRF sites that store non-combustible, recyclable, waste material: Financial assurance per tonne of recyclable waste= Gate fees of the landfill per tonne+Transport costs per tonne+EPA administration charges where: - Gate fees of the landfill = $200.00 per tonne - Transportation costs = $15.00 per tonne Administration charges are calculated based on the following assumptions: - the salary for a project manager for the number of months it would take to remove the waste stored at the facility, according to the permission conditions - the monthly project manager salary of $9,113 - the amount of waste that can be removed in one month is 4,000 tonnes

The formula for financial assurance for WRRF sites that store combustible, recyclable, waste material: Financial assurance per tonne of recyclable waste= Gate fees of the landfill per tonne+Transport cost per tonne+EPA administration charges ×1.5 where: - Gate fees of the landfill = $200.00 per tonne - Transportation costs = $15.00 per tonne Administration charges are calculated based on the following assumptions: - the salary for a project manager for the number of months it would take to remove the waste stored at the facility, according to the permission conditions - the monthly project manager salary of $9,113 - the amount of waste that can be removed in one month is 4,000 tonnes

# Calculating financial assurance for bulk storage

Financial assurance for bulk storage facilities is based on a calculation of site-specific costs to clean up the site. The costs are based on a scenario where environmental or human health risks and/or issues result from a fire and the subsequent abandonment of the site.

On-site or off-site contamination of land or groundwater of a residual nature, for which a remediation cost allowance in the calculation can be made, is to be included.

The financial assurance may be used for clean up or remediation where the duty holder fails to undertake their obligations.

## Activities and assumptions for calculating financial assurance for bulk storage

The following activities are to be considered when calculating clean up costs for the fire component of the clean up scenario:

  - Environmental clean up

  - Collection of firefighting solutions

  - Treatment of firefighting solutions

  - Residual hydrocarbon removal to slops tanks

  - Treatment of residual hydrocarbon material

  - Make safe works (e.g. works to address immediate safety risks of damaged tanks)

The activities to be considered when calculating clean up costs for the abandonment component of the clean up scenario are:

  - General environmental clean up (assumes clean up of tanks); and

  - Make safe works (e.g. investigations, clean down of pipelines and valves, removal of pumps, etc.).

The allowances for general environmental clean up and make safe works may be refined where decommissioning plans are available for the site.

The following assumptions are made for the fire and abandonment scenario:

  - Tanks have been fully emptied before abandonment of the site.

  - Hydrocarbon fuels were stored in tanks during a fire event.

  - Residual material may be present in pipelines and associated infrastructure after abandonment.

  - A fire event resulted in minor to medium damage to tank infrastructure.

  - Residual (i.e. the near surface soil layer beneath a concrete slab) contamination of soil only has occurred.

## Site specific considerations and minimum cost information

Table 11 and Table 12 outline the activities and minimum information for calculating the clean up costs.

For fire, site-specific risk assessment should inform the number of tanks for inclusion in the costing. Clean up costs increase with additional tanks or higher storage capacity (combined). At a minimum, the cost calculation should include the largest tank on-site and associated clean up efforts. The quantity of expected firefighting run-off, residual hydrocarbon material and potential environmental impacts should be informed by:

  - site-specific safety case studies

  - emergency plans

  - associated documentation.

As a conservative approach, the quantity of firefighting run-off should consider secondary events (e.g. rain).

The abandonment cost component should include the works to clean up the tanks on the site not associated with a fire event.

Site-specific circumstances to determine the appropriate amount of allowance for the clean up program indirect costs should be considered. This will include, for example:

  - proximity and potential impacts to sensitive receptors (e.g. environmental values, community and neighbouring business)

  - material stored on-site

  - potential release of secondary pollutants during fire requiring onsite and off-site monitoring.

Project management and administrative costs are to be estimated at 10% of clean up costs. This may be reduced in instances where this amount is not proportionate to the program requirement. For example, where there may be economies of scale, or the work is routine in nature.

The cost estimates should use rates and costs considered representative at the time at which the estimation is prepared. A contingency amount is to be included to address the uncertainties associated with the clean up program costs. The rationale for the contingency amount should be provided.

Higher contingency is expected where estimates are prepared based on very limited information regarding:

  - site-specific risks

  - estimates are provided at assembly level of detail rather than individual unit cost line items, and/or

  - low-quality information is provided to support quantities, unitary costs and rates.

Table 11	Minimum cost information for bulk storage facility clean up – fire cost component

| Clean up program activity | Clean up task and description 1 | Unit | Quantity | Rate ($/unit) | Estimated Cost ($) | % of total cost |
| --- | --- | --- | --- | --- | --- | --- |
| Direct clean up activities |  |  |  |  |  |  |
| Removal of contained liquids impacted by firefighting solutions | As a minimum, the following must be included as relevant: - Estimate of contained liquids impacted by firefighting solutions - Number of tankers required to collect liquids impacted by firefighting solutions - Personnel and equipment |  |  |  |  |  |
| Treatment and/or disposal of liquids impacted by firefighting solutions | As a minimum, the following must be included as relevant: - Estimate of contained liquids impacted by firefighting solutions - Transportation to treatment and/or disposal facility - Treatment and/or disposal costs |  |  |  |  |  |
| Removal of residual hydrocarbons or other residual product from tanks damaged by fire and/or within bunded areas | As a minimum, the following must be included as relevant: - Estimate amount of residual product - Personnel and equipment |  |  |  |  |  |
| Treatment or disposal of removed residual hydrocarbon or other residual treatment (onsite or offsite) | As a minimum, the following must be included as relevant: - Estimate amount of residual product - Transportation to treatment and/or disposal facility - Treatment and/or disposal costs |  |  |  |  |  |
| Environmental | As a minimum, the following must be included as relevant: - Personnel and equipment - Removal of ash residue, dust, smoke and soot - Estimate amount of waste - Collection of waste - Disposal of waste (including transportation) - Estimate amount of contaminated concrete slab and/or soil (in the case of earthen bunds) within the bunded area associated with the fire scenario - Removal of contaminated concrete slab and/or soil (bunded areas) - Lawful disposal of contaminated concrete slab (including transportation) - Lawful disposal of contaminated soil (including transportation) - Treatment of exposed soils to prevent dust emissions and minimise vegetation growth - Drainage works |  |  |  |  |  |
| Make safe works - Large tanks - Medium tanks - Small tanks | As a minimum, the following must be included as relevant: - Personnel and equipment - Description of tanks onsite - Removal and/or repair of unstable tank structures (impacted by the fire) - Disposal of waste (including transportation) Note that works are limited to address immediate safety risks posed by fire damaged tanks (e.g. unstable structure, leaking etc.), and excludes remedial works required for business continuance. |  |  |  |  |  |
| Contractor management | As a minimum, the following must be included as relevant: - Coordination of specialised contractors - Supervision of mobilisation and demobilisation works - Supervision of health, safety and security procedures - Supervision of work approach |  |  |  |  |  |
| Associated activities with clean up |  |  |  |  |  |  |
| Consulting engineering | As a minimum, the following must be included as relevant: - Site investigation and assessment of fire damage (structural, electrical, water and gas) - Sampling and testing (structural, waste characterisation, dangerous goods, explosive atmosphere etc.) - Traffic management - Site security |  |  |  |  |  |
| Miscellaneous environmental costs | As a minimum, the following must be included as relevant: - Onsite and offsite air quality monitoring (particulates, VOCs etc.) - Stormwater management - Ongoing monitoring, in addition to routine monitoring undertaken at the site as per the Risk Management and Monitoring Program for the site |  |  |  |  |  |
| SUBTOTAL |  |  |  |  |  |  |
| EPA project management and administrative costs | Calculated at 10% of total clean up costs |  |  |  |  | 10 |
| Contingency | Provide the rationale for contingency |  |  |  |  |  |
| TOTAL ESTIMATED COST |  |  |  |  |  |  |

Notes:

1 – The assumptions made for the purpose of the calculation of costs must be clearly outlined and should refer to the supporting information used to generate the quantity and rate. Where the program activity encompasses multiple tasks, these should be calculated individually where practicable.

Table 12	Minimum cost information for bulk storage facility clean up – abandonment cost component

| Clean up program activity | Clean up task and description 1 | Unit | Quantity | Rate ($/unit) | Estimated Cost ($) | % of total cost |
| --- | --- | --- | --- | --- | --- | --- |
| Direct clean up activities |  |  |  |  |  |  |
| Environmental - Targeted clean-up of tanks to address risks | As a minimum, the following must be included as relevant: - Personnel and equipment - Estimate amount of residual hydrocarbons (or other) product in tanks - Removal of residual hydrocarbon (or other) product in tanks - Decontamination / clean up of tanks - Estimate amount of contaminated concrete slab and/or soil within the bunded areas associated with the abandoned tanks. - Removal of contaminated concrete slab or soil (bunded areas) - Estimate amount of waste - Disposal of waste (including transportation) - Disposal of contaminated concrete slab (including transportation) - Disposal of residual contaminated soil (including transportation) - Treatment of exposed soils to prevent dust emissions and minimise vegetation growth |  |  |  |  |  |
| Make safe works - Large tanks - Medium tanks - Small tanks | As a minimum, the following must be included as relevant: - Personnel and equipment - Estimate residual hydrocarbon (or other) product in pipelines, valves, etc. - Removal of residual hydrocarbon (or other) product in pipelines, valves, etc. - Decontamination / clean up of pipelines, valves etc. - Estimate quantity of ancillary infrastructure for removal - Removal of ancillary infrastructure posing environmental and/or safety risks (e.g., electrical risks and collapsing structures) - Disposal of waste (including transportation) |  |  |  |  |  |
| Contractor Management | As a minimum, the following must be included as relevant: - Coordination of specialised contractors - Supervision of mobilisation and demobilisation works - Supervision of health, safety and security procedures, confined space entry, etc. - Supervision of work approach |  |  |  |  |  |
| Associated activities with clean up |  |  |  |  |  |  |
| Consulting engineering | As a minimum, the following must be included as relevant: - Site investigation and assessment (structural, electrical, water and gas) - Sampling and testing (structural, waste characterisation, dangerous goods, explosive atmosphere etc.) - Traffic management - Site security |  |  |  |  |  |
| Miscellaneous environmental costs | As a minimum, the following must be included as relevant: - Ongoing monitoring, in addition to routine monitoring undertaken at the site as per the Risk Management and Monitoring Program for the site |  |  |  |  |  |
| SUBTOTAL |  |  |  |  |  |  |
| EPA project management and administrative costs | Calculated at 10% of total clean up costs |  |  |  |  | 10 |
| Contingency | Provide the rationale for contingency |  |  |  |  |  |
| TOTAL ESTIMATED COST |  |  |  |  |  |  |

Notes:

1 – The assumptions made for the purpose of the calculation of costs must be clearly outlined and should refer to the supporting information used to generate the quantity and rate. Where the program activity encompasses multiple tasks, it is recommended that these are calculated individually where practicable.

# Calculating financial assurance for contaminated sites – on-site soil retention

Financial assurance may be required for the activity of contaminated sites - on-site soil retention. This is the containment of contaminated soil (other than fill material) on-site in a structure designed to contain at least 1,000 m3 of the soil and to prevent further contamination.

This permission activity is a development activity only. The calculation of financial assurance is based on the site-specific costs for the design and construction of the contaminated soil containment infrastructure. This is because the clean up activity at this type of site is expected to be the completion of the EPA approved development activity works to manage the risk through on-site retention of the contaminated soil.

## Assessment by an environmental auditor

We require an independent assessment of the amount of financial assurance for contaminated sites - on-site soil retention activities to be provided by an environmental auditor, appointed under the Act.

Find more information about auditor assessment of financial assurance proposals, and the assessment form on our website https://www.epa.vic.gov.au/financial-assurance-permissions-and-contaminated-land.

## Activities for calculating financial assurance for contaminated sites - on-site soil retention

The following activities for on-site site retention are to be considered in the cost estimation:

  - Soil sampling and assessment

  - Storage structural testing

  - EPA project management and administration

  - Containment structure design

  - Containment structure construction

  - Risk management plan

  - Environmental auditor

## Site specific considerations and minimum cost information

Table 13 outlines the activities and minimum information for the clean up cost estimation.

The applicability of the site-specific cost estimation elements will be defined in accordance with the development licence requirements. For example, if sufficient soil sampling assessment has been completed, soil sampling and assessment will not be required.

Table 13	Minimum cost information for contaminated sites – on-site soil retention

| Program activity | Task description | Unit | Quantity | Rate ($/unit) | Estimated Cost ($) | % |
| --- | --- | --- | --- | --- | --- | --- |
| Soil sampling and assessment | Soil sampling and assessment to determine the contaminant(s), contaminant(s) concentrations, contaminant(s) leachability and appropriate containment design Example of costs to be included: - Sampling consumables - Equipment (e.g., drilling rig, backhoe, excavator, auger, analysers etc.) - Professional fees (sampling, data interpretation and reporting) - Laboratory costs |  |  |  |  |  |
| Storage structural testing | Testing for bearing pressure and geotechnical stability for containment features Example of costs to be included: - Sampling equipment - Professional fees (sampling, data interpretation and reporting) |  |  |  |  |  |
| Containment structure design | Example of costs to be included: - Professional fees |  |  |  |  |  |
| Containment structure construction - mobilisation - site establishment - soil excavation - construction materials - groundwater management and treatment related to excavation - waste disposal - demobilisation - other construction activities | Example of costs to be included: - Materials - Equipment - Professional fess - Construction management - Waste transportation fees - Waste disposal fees - Construction quality assurance (CQA) testing for materials - CQA supervision (e.g., level 1 geotechnical inspection and testing authority) |  |  |  |  |  |
| Risk management plan | Risk management plan to satisfy licence conditions and/or environmental duties required under the Environment Protection Regulations 2021. - Example of costs to be included: - Preparation of initial risk management plan - Annual updates of the risk management plan |  |  |  |  |  |
| Environmental auditor (as required by the development licence) | EPA appointed environmental euditor fees, including revision and verification of structure design and related documents, and/or verification of as-built structure in accordance EPA approved designs. - Example of costs to be included: - Professional fees for auditing and preparation of an audit report |  |  |  |  |  |
| EPA project management and administrative costs | Planning and other administrative tasks, including project management, tendering, contract administration etc. - Example of costs to be included: - Salary of a project manager for the number of months it would take to complete the construction of the L02 containment structure. |  |  |  |  |  |
| TOTAL ESTIMATED COST |  |  |  |  |  |  |

# Appendix A – Calculating a Consumer Price Index adjustment for financial assurance – reportable priority waste management

CPI means the Consumer Price Index (All Groups Index) issued by the Australian Statistician for Melbourne.

The following formula is used when calculating financial assurance after 2015. N=E(1 +((A-B)/B)) where: N is the updated financial assurance calculation E is the financial assurance calculated in 2015 dollars A is the CPI number for the previous December quarter of the year that the calculation is being performed B is the CPI number for the December quarter of 2015

# Appendix B – Calculating a Consumer Price Index adjustment for financial assurance – waste resource recovery facilities

CPI means the Consumer Price Index (All Groups Index) issued by the Australian Statistician for Melbourne.

The following formula is used when calculating financial assurance after 2021. N-E(1 +((A-B)/B)) where: N is the updated financial assurance calculation E is the financial assurance calculated in 2021 dollars A is the CPI number for the previous December quarter of the year that the calculation is being performed B is the CPI number for the December quarter of 2021

Accessibility

Contact us if you need this information in an accessible format such as large print or audio. 
Please telephone 1300 372 842 or email contact@epa.vic.gov.au

Interpreter assistance

If you need interpreter assistance or want this document translated, please call 131 450 and advise your preferred language. If you are deaf, or have a hearing or speech impairment, contact us through the National Relay Service.

epa.vic.gov.au

Environment Protection Authority Victoria

GPO BOX 4395 Melbourne VIC 3001

1300 372 842

The cost of a hydrogeological assessment is not required where all cells onsite are fully engineered to the standard set out in Siting, design, operation and rehabilitation of landfills (publication 788) (https://www.epa.vic.gov.au/about-epa/publications/788-3).

The cost of establishing an environmental monitoring program is not required in the closure and aftercare financial assurance estimate if this program is already in place under Landfill licensing (publication 1323) (https://www.epa.vic.gov.au/about-epa/publications/1323-3).

The cost of preparing a rehabilitation plan and an aftercare management plan is not required in the closure and aftercare financial assurance estimate if the documents are already in place with Siting, design, operation and rehabilitation of landfills (publication 788) (https://www.epa.vic.gov.au/about-epa/publications/788-3).